UNIVERSITI TUNKU ABDUL RAHMAN

FACULTY OF INFORMATION & COMMUNICATION TECHNOLOGY

BACHELOR OF INFORMATION TECHNOLOGY (HONS)

COMPUTER ENGINEERING

UCCE1064 PROCESSOR AND INTERFACING

TITLE: Servo Motor Control Module

| No. | Name | Student ID |
| --- | --- | --- |
| 1. | Ng Jing Xuan | 23ACB02444 |
| 2. | Ng Yu Heng | 23ACB02307 |
| 3. | Chow Bin Lin | 23ACB01105 |

Contents

1 General Information and Product Specifications	2

1.1	General Information	2

1.2	Product Specifications	2

2 Hardware and Software Partition	3

2.1 Hardware Partition	3

2.2 Software Partition	5

3 Algorithm	6

3.1 Declaration	6

3.2 Pseudocode	6

4 Screenshots of final product	11

5 References	12

1   General Information and Product Specifications

  - General Information

A servo motor is a type of motor which consists of a control circuit that provides feedback on the current position of the motor shaft. This feedback allows the servo motor to rotate an object with great precision at some specific angles (Apoorve,2025). Thus, a Servo Motor Control Module is designed to control a servo motor by using 8051 microcontroller-based development board.

The design in this assignment allows user to choose multiple speed modes for the servo motor through a keypad interface. After selected a speed mode, user can choose confirm to start the servo motor or choose to reset through the same keypad to select another speed mode. When the servo motor is running, LCD will display the current speed mode while two 7-segment display will show the number of complete servo cycle as real-time feedback. The servo motor will move forward and backward with predefined delay times according to the selected speed mode. After the servo motor has completed 10 cycles, user can choose to start again with the same speed mode or reset speed mode. There are also push buttons for emergency stop and reset function to interrupt the running of servo motor.

  - Product Specifications

The following are the functionalities provided by the product.

- LCD Menu Interface: Displays user prompts and current speed mode selection.

- Speed Mode Selection: Four predefined modes to control movement speed of servo motor

- Speed Modes Specification:

Speed Mode	Sequence				      Wait Time (Seconds)

0	Backward → Wait → Forward → Wait	3s

1	Backward → Wait → Forward → Wait	2.5s

2	Backward → Wait → Forward → Wait	2s

3	Backward → Wait → Forward → Wait	1s

- Cycle Counter: Counts and displays how many cycles the servo motor has completed (maximum 10 counts).

- Emergency Stop button: Immediately stops the motor and returns to the main menu.

- Reset button: Returns to main menu but allows the servo motor to continue running.

- Confirm / Start button: To confirm the speed mode selection or to start the servo motor again after finish.

2   Hardware and Software Partition

2.1 Hardware Partition

Figure 2.1.1 Block diagram of Servo Motor Control Module

| No. | Block | Description |
| --- | --- | --- |
| 1 | 8051 MCU (89V51RD2 Microcontroller Unit) | - Use to interfaces with 4x4 hexadecimal keypad, LCD, servo motor, 7-segment display, and push buttons. - Connection: - P0 → LCD (DB0-DB7) - P1.0-P1.5 → 4x4 Hexadecimal Keypad (R1, R2, C1-C4) - P2 → 7-segment display (CN2) - P3.0, P3.1, P3.4 → LCD (EN, R/W, RS) - P3.5 → Servo Motor PWM - P3.6, P3.7 → 7-segment display (CN1 - 0, 1) |
| 2 | 4x4 Hexadecimal keypad | - Used by the user to select the speed mode (Speed 0, 1, 2, 3), confirm or reset. - Connection: - R1, R2 → 8051 MCU (P1.0 – P1.1) - C1-C4, 8051 MCU (P1.2 - P1.5) |
| 3 | Push buttons | - Shows the number of completed back-and-forth cycles (up to 10). - Connection: - SW1 → 8051 MCU (P3.2) - SW4 → 8051 MCU (P3.3) |
| 4 | 7-segment display | - Shows the number of completed back-and-forth cycles (0 to 10). - Connection: - CN2 - 0, 1 → 8051 MCU (P1.6, P1.7) - CN4 → 8051 MCU (P2) |
| 5 | LCD display | - Displays the menu options, current speed mode, prompt for confirmation/reset, and system status. - Connection: - DB0-DB7 → 8051 MCU (P0) |
| 6 | Servo motor | - Rotates back (from 0° to 180°) and forth (from 180° to 0°) based on speed mode selected. - Connection: - Vcc → 5V DC power - Ground → GND - PWM → P3.5 |
| 7 | COM Port & RS-232 | - Used for uploading the hex code from PC to 8051 - Connection: - USB→RS-232→ 8051 MCU (UART) |
| 8 | DC Power supply | - Powers the 8051 board and servo motor. - Connection: - Power supply adapter →Power supply voltage regulator |

Table 2.1 Table  of Servo Motor Control Module Hardware partition

2.2 Software Partition

| No. | Module | Description |
| --- | --- | --- |
| 1 | - Main module | - Initializes the system - Displays menu - Waits for user input - Assign selected speed mode base on keypad press - Confirm or reset selected speed mode - Emergency stop and reset the system |
| 2 | LCD Display module | - Display user options - Display speed mode |
| 3 | Keypad Scanning module | - Continuously scans rows/columns - Debounces input - Maps pressed key to corresponding command |
| 4 | Servo Motor Control module | - Generates PWM signal using Timer 0 interrupts - Sets pulse width based on target angle - Call delay base on selected speed after half cycle rotation - Count cycle count for each full cycle |
| 5 | Interrupt module | - Use timer 0 interrupt to handle PWM high and low cycle - Use external interrupt 0 to detect emergency stop button pressed - Use external interrupt 1 to detect reset button pressed |
| 6 | 7-Segment Display module | - Decode cycle count into 7-segment display code - Display both digits on 7-segment display - Multiplexing display 0 and display 1 with proper delay |
| 7 | Emergency stop and reset module | - Wait for push button inputs - Stop the system when emergency stop button pressed - Reset 7-segment display and LCD when reset button is pressed |
| 8 | Delay module | - Generate delay for LCD to display - Use timer 1 to generate delay for servo motor base on different speed mode chosen - Generate delay for 7-segment display multiplexing |

Table 2.2 Table of Servo Motor Control Module Software partition

3   Algorithm

3.1 Declaration

| Name | Value/ Port Memory address | Function |
| --- | --- | --- |
| SPEED0 | 60 | Loop count for the servo motor to wait. (60*0.05s = 3s) |
| SPEED1 | 50 | Loop count for the servo motor to wait. (50*0.05s = 2.5s) |
| SPEED2 | 40 | Loop count for the servo motor to wait. (40*0.05s = 2s) |
| SPEED3 | 20 | Loop count for the servo motor to wait. (30*0.05s = 1s) |
| STOP_FLAG | 23H.0 | To check if the emergency stop button is pressed |
| RESET_FLAG | 23H.1 | To check if reset button is pressed |
| MOTOR_RUNNING | 23H.2 | To check the motor running or not |
| CHOOSE_MSG | 21H | Choose which message to display |
| DEBOUNCE | 22H | To store the debounce times |
| SPEED_MODE | 30H | A temporary memory to store the loop count |
| CYCLE_COUNT | 31H | A temporary memory to store the number of cycles had been ran |
| PWM_FLAG | 32.1 | Flag to indicate PWM high/ low signal |
| PWM_PIN | P3.5 | PWM output pin for servo motor |
| SEG_PORT | P2 | Used for display 7 segment display code |
| DIS0 | P3.6 | To select display 0 |
| DIS1 | P3.7 | To select display 1 |
| RS | P3.4 | Register select, data input or command input |
| RW | P3.1 | decide to write/read to the LCD |
| EN | P3.0 | Enable to activate read or write function to the LCD |

3.2 Pseudocode

3.2.1 Main module START Locate to RAM location 0000H Jump to MAIN // interrupt initialization Locate to RAM location 0003H // ISR External interrupt 0 Jump to ISR_INT_PWM if ISR External interrupt 0 has been triggered Locate to RAM location 000BH // Timer 0 interrupt Jump to TIMER0_INT_PWM if timer 0 interrupt has been triggered Locate to RAM location 0013H // ISR 0 External interrupt 1 Jump to ISR_INT1 if ISR External interrupt 1 has been triggered Located to RAM location 0050H MAIN: enable external interrupt0,1and timer 0 interrupt Set timer 0 and timer 1 to mode 1 Clear RW to write Call INITIALIZE_LCD Display startup message START_AGAIN: Iteration Initialize the variables and motor reset to 0 degree LCD_RESET: Call CLEAR_LCD Display “Select Speed” Display “0, 1, 2 OR 3” WAIT_MOTOR: If the motor is running Jump to CALL_MOTOR Call KSCAN1 to wait for user to choose the speed mode Call CLEAR_LCD Display which mode is selected, “Mode X Selected” Move to the second line of LCD Display “Confirm or Reset” Call function KSCAN2 to scan user press confirm or reset button If the reset button is pressed, Repeat the iteration else Continue to the next step SERVO_MOTOR_AGAIN: Iteration Display “Servo Motor”, Display which mode is running, “Mode X Running..” Call SERVO Call CLEAR_LCD Display “Running Complete” Display “Start or Reset?” Call function GO_ADDRESS Call function KSCAN2 to can which button user select start again or reset If the reset button is pressed Go to START_AGAIN else Repeat the iteration CALL_MOTOR: Call SERVO If motor is running Jump to WAIT_MOTOR 3.2.2 LCD Display Module INITIALIZE_LCD: Send command 38H (Function Set) Send command 0FH (Display ON) Send command 06H (Entry Mode Set) Return CLEAR_LCD: Send command 01H (Clear display) Return WRITE_STRING: Set R0 to 0 Iteration Get character from address in DPTR + R0 If character is zero: Go to EXIT Else: Call WRITE_CHAR with character Increment R0 Repeat the iteration EXIT: Return GO_ADDRESS: Send command C0H (Set DDRAM address to 2nd line) Return WRITE_CMD: Write A to P0 SET RS to 0 Set EN to 1 then set it to 0 Call LCD_DELAY Return WRITE_CHAR: Write A to P0 SET RS to 1 Set EN to 1 then set it to 0 Call LCD_DELAY Return 3.2.3 Keypad Scanning Module KSCAN1: Wait until keypad first row button is pressed (ROWRD1) Debounce input Get which column user is press and set the SPEED_MODE and CHOOSE_MSG (COLRD1) Return KSCAN2: Wait until keypad second row button is pressed (ROWRD2) Debounce input Get user press confirm or reset (COLRD2) Call COLRD2 Return 3.2.4 Servo Motor Control Module SERVO: Start timer 0 Motor is running Clear STOP_FLAG and RESET_FLAG If CYCLE_COUNT is not 0 Go to NEW_CYCLE Call SERVO_DELAY If STOP_FLAG is high Go to EMERGENCY_STOP If RESET_FLAG is high Go to RESET NEW_CYCLE: Iteration: Call DIS_7SEG ;DEGREE180 (backwards): Set Timer 0 high duration value: 2.4ms (R5, R6) Set Timer 0 low duration value: 17.6ms (R3, R4) Call SERVO_DELAY If STOP_FLAG is high Go to EMERGENCY_STOP If RESET_FLAG is high Go to RESET ;DEGREE0 (forward): Set Timer 0 high duration value: 0.5ms (R5, R6) Set Timer 0 low duration value: 19.5ms (R3, R4) Call SERVO_DELAY If STOP_FLAG is high Go to EMERGENCY_STOP If RESET_FLAG is high Go to RESET Increment CYCLE_COUNT If CYCLE_COUNT <10 Continue the iteration Motor is not running RET 3.2.5 Interrupt Module TIMER0_INT_PWM: If PWM_FLAG is high Go to LOW_CYCLE HIGH_CYCLE: SET PWM_FLAG to 1 SET PWM_PIN to 1 Load timer 0 (R5, R6) Clear TF0 Return from interrupt LOW_CYCLE: SET PWM_FLAG to 0 SET PWM_PIN to 0 Load timer 0 (R3, R4) Clear TF0 Return from interrupt ISR_INT0: Set STOP_FLAG to 1 Return from interrupt ISR_INT1: Set RESET_FLAG to 1 Return from interrupt 3.4.6 7-Segment Display Module DIS_7SEG: Convert CYCLE_COUNT to decimal Display CYCLE_COUNT on 7 segment display Return 3.4.7 Emergency Stop and Reset Module EMERGENCY_STOP: Stop Timer 0 Clear STOP_FLAG Clear PWM_PIN Clear TF0 Go to START_AGAIN RESET: Clear RESET_FLAG Call LCD_RESET Go to SERVO 3.4.8 Delay Module LCD_DELAY: Load timer 1(DBF0H) Activate timer 1 Wait until TF1 = 1 then move to the next step Stop timer 1 and clear TF1 Return DELAY_1s: - Return SERVO_DELAY: Repeat the SPEED_MODE times begin Call DIS_7SEG Load timer 1(4BFDH) Start timer 1 Loop begin Call DIS_7SEG End if TF1 = 1 Stop timer 1 and clear TF1 RET

4   Screenshots of final product

Figure 4.1 Connection of ports and wires                        Figure 4.2 Connection of ports and wires

Figure 4.3 System startup message                                 Figure 4.4 System startup message (2)

Figure 4.5 Selecting speed message                                Figure 4.6 Confirmation message

Figure 4.7 Servo motor running message

5   References

Apoorve (2025). What is a Servo Motor? - Understanding the basics of Servo Motor Working. [online] Circuit Digest. Available at: https://circuitdigest.com/article/servo-motor-working-and-basics.

‌